| OPERATIONAL MANUAL |  |  |  |
| --- | --- | --- | --- |
| #### Windows 98 / 2000 / NT / XP ### Communication ## Driver # TWSWTCP.EXE |  |  |  |
|  | - EDITION |  |  |
|  |  | Month | Year |
|  | 3rd | August | 2000 |
|  | 4th | May | 2001 |
|  | 5th | April | 2002 |
|  | 6th | August | 2002 |
| TERAOKA WEIGH-SYSTEM PTE LTD 4 LENG KEE ROAD , #05-03/04/05 , SIS BUILDING , SINGAPORE 159088. TEL: (65) 6472 2996 / 6378 5361 FAX: (65) 6472 2869 |  |  |  |

CONTENTS

- SM-90 WORKSTATION CONFIGURATION..………………………….……3

- WINDOWS 98 / 2000 / NT / XP & SM-90 COMMUNICATION

2.1 ETHERNET ADAPTER CARD	4

2.2 HOSTS	4

2.3 SCALE PORT NUMBER	4

2.4 SM-90 FILES SUPPORTED BY TWSWTCP.EXE	5

2.5 ERROR CODES GENERATED BY TWSWTCP.EXE	8

2.6 UPLOAD PLU ITEM FILE	9

2.7 UPLOAD PLU DELETE FILE	11

2.8 DOWNLOAD PLU ITEM FILE	12

2.9 DOWNLOAD PLU TOTAL FILE	13

2.10 UPLOAD PRESET KEY ASSIGNMENT FILE	15

SM-90 WORKSTATION CONFIGURATION WITH PC (ETHERNET INTERFACE)

Workstation #1    Workstation #2     Workstation #3

………..

PC

HUB

NOTE :	Maximum of 255 SM-80SX / SM-90 scales can be connected via the above

configuration and an Ethernet Adapter Card has to be installed in the PC.

Downloading of data to individual scale one by one.

SPEC setting on scale :

SPEC 49	CLIENT / SERVER INTERFACE

0	No interface 				3	Not used

1	Ethernet - Coaxial cable		4	4 lines, RS485 (For future use)

2	Ethernet - Twisted cable

SPEC 50	CLIENT / SERVER

0	Client					2	Back-up server

1	Server / Workstation

SPEC 135	PORT NUMBER

* Enter number from 1 to 255

NOTE : Please clear the memory ( [REZERO]+[FEED] when power on ) of the scale

after setting the SPEC for the Ethernet Server / PC connection.

WINDOWS O/S & SM-90 COMMUNICATION VIA TWSWTCP.EXE

PC ETHERNET ADAPTER CARD

Start  Settings  Control Panel  Network  TCP / IP ( Select Properties )

Configure the IP Address and Submask of the PC ethernet adapter card as follows :

IP Address : 192.168.0.XXX where XXX can be any number from 0 to 255

Submask : 255.255.255.0

HOSTS

Text file which contains the IP Addresses of all the SM-80SX / SM-90 scales which is connected to the scale network.

WINDOWS 98 Operating System

HOSTS is located in Windows folder

WINDOWS 2000 / XP Operating System

HOSTS is located in Windows\System32\Drivers\Etc folder

WINNT Operating System

HOSTS is located in Winnt\System32\Drivers\Etc folder

Sample of HOSTS :

192.168.0.1	S0001

192.168.0.2	S0002

192.168.0.3	S0003

SCALE PORT NUMBER

TWSWTCP.EXE driver uses the following scale port number via WinSocks :

2000  +  (Port number specified in Spec 135)

Example :  Scale IP Address = 192.168.0.3

Scale Port Number = 2003

##### SM-90 FILES SUPPORTED BY TWSWTCP.EXE

The list of SM-90 files supported by TWSWTCP.EXE is as follows:

UPLOAD FILES ( PC  SM-80SX / SM-90 )

| FILE NAME | COMMAND | UPLOAD DATA FILE |
| --- | --- | --- |
| Department | # TWSWTCP F20.DAT 1 | SM001F20.DAT |
| Clerk | # TWSWTCP F22.DAT 1 | SM001F22.DAT |
| Main Group | TWSWTCP F23.DAT 1 | SM001F23.DAT |
| PLU | TWSWTCP F25.DAT 1 | SM001F25.DAT |
| Print Format | TWSWTCP F34.DAT 1 | SM001F34.DAT |
| Scale Data | TWSWTCP F37.DAT 1 | SM001F37.DAT |
| Text | TWSWTCP F38.DAT 1 | SM001F38.DAT |
| Scroll Message | TWSWTCP F39.DAT 1 | SM001F39.DAT |
| Scroll Sequence | TWSWTCP F40.DAT 1 | SM001F40.DAT |
| Preset Key | TWSWTCP F41.DAT 1 | SM001F41.DAT |
| Production Place | TWSWTCP F42.DAT 1 | SM001F42.DAT |
| Advertisement | TWSWTCP F43.DAT 1 | SM001F43.DAT |
| Password | TWSWTCP F44.DAT 1 | SM001F44.DAT |
| Slaughter House | TWSWTCP F45.DAT 1 | SM001F45.DAT |
| Country | TWSWTCP F47.DAT 1 | SM001F47.DAT |
| Ingredient | TWSWTCP F50.DAT 1 | SM001F50.DAT |
| Special Message | TWSWTCP F51.DAT 1 | SM001F51.DAT |
| Shop Name | TWSWTCP F52.DAT 1 | SM001F52.DAT |
| Cutting Hall | TWSWTCP F54.DAT 1 | SM001F54.DAT |
| Traceability | TWSWTCP F56.DAT 1 | SM001F56.DAT |
| Traceability Record (DPS90) | TWSWTCP F58.DAT 1 | SM001F58.DAT |
| Traceability Report (DPS90) | TWSWTCP F63.DAT 1 | SM001F63.DAT |
| Image (SM700) | TWSWTCP F10.DAT 1 | SM001F10.DAT |
| Department 2 (SM700) | TWSWTCP F14.DAT 1 | SM001F14.DAT |
| PLU 2 (SM700) | TWSWTCP F17.DAT 1 | SM001F17.DAT |

Note: It is necessary to create the UPLOAD DATA FILE for each individual scale before using the TWSWTCP.EXE driver to upload the respective UPLOAD DATA FILES to the scales.

DELETE FILES ( PC  SM-80SX / SM-90 )

| FILE NAME | COMMAND | DELETE DATA FILE |
| --- | --- | --- |
| Department | # TWSWTCP F90.DAT 1 | SM001F90.DAT |
| Main Group | TWSWTCP F93.DAT 1 | SM001F93.DAT |
| Preset Key | TWSWTCP F94.DAT 1 | SM001F94.DAT |
| PLU ( Delete Selected Items ) | TWSWTCP F95.DAT 1 | SM001F95.DAT |
| PLU ( Delete All Items ) | TWSWTCP F88.DAT 1 |  |
| Traceability Report | TWSWTCP F73.DAT 1 | SM001F73.DAT |
| Traceability Record | TWSWTCP F75.DAT 1 | SM001F75.DAT |
| Traceability | TWSWTCP F76.DAT 1 | SM001F76.DAT |
| Cutting Hall | TWSWTCP F77.DAT 1 | SM001F77.DAT |
| Country | TWSWTCP F81.DAT 1 | SM001F81.DAT |
| Slaughter House | TWSWTCP F82.DAT 1 | SM001F82.DAT |
| Advertisement | TWSWTCP F84.DAT 1 | SM001F84.DAT |
| Clerk | TWSWTCP F86.DAT 1 | SM001F86.DAT |
| Production Place | TWSWTCP F87.DAT 1 | SM001F87.DAT |
| Print Format | TWSWTCP F89.DAT 1 | SM001F89.DAT |
| Text | TWSWTCP F98.DAT 1 | SM001F98.DAT |
| Scroll Message | TWSWTCP F99.DAT 1 | SM001F99.DAT |
| Scroll Sequence | TWSWTCP F97.DAT 1 | SM001F97.DAT |
| Shop Name | TWSWTCP F92.DAT 1 | SM001F92.DAT |
| Ingredient | TWSWTCP F96.DAT 1 | SM001F96.DAT |
| Special Message | TWSWTCP F91.DAT 1 | SM001F91.DAT |
| Shop Name | TWSWTCP F92.DAT 1 | SM001F92.DAT |
| Image (SM700) | TWSWTCP F12.DAT 1 | SM001F12.DAT |
| Department 2 (SM700) | TWSWTCP F15.DAT 1 | SM001F15.DAT |
| PLU 2 (SM700) | TWSWTCP F18.DAT 1 | SM001F18.DAT |

Note: It is necessary to create the DELETE DATA FILE for each individual scale before using the TWSWTCP.EXE driver to upload the respective DELETE DATA FILES to the scales. However, this is not necessary if this command TWSWTCP F88.DAT 1 is used to delete all PLU items in the scale.

DOWNLOAD FILES ( PC  SM-80SX / SM-90 )

| FILE NAME | COMMAND | DOWNLOAD DATA FILE |
| --- | --- | --- |
| # Department | # TWSWTCP F80.DAT 1 | SM001T80.DAT |
| # Department Total | # TWSWTCP F21.DAT 1 | SM001T21.DAT |
| Main Group | TWSWTCP F83.DAT 1 | SM001T83.DAT |
| Main Group Total | TWSWTCP F24.DAT 1 | SM001T24.DAT |
| PLU | TWSWTCP F85.DAT 1 | SM001T85.DAT |
| PLU Total | TWSWTCP F26.DAT 1 | SM001T26.DAT |
| Scale Data | TWSWTCP F27.DAT 1 | SM001T27.DAT |
| Clerk | TWSWTCP F28DAT 1 | SM001T28.DAT |
| Clerk Total | TWSWTCP F29.DAT 1 | SM001T29.DAT |
| PLU Total -Transaction Type | TWSWTCP F30.DAT 1 | SM001T30.DAT |
| Production Place | TWSWTCP F31.DAT 1 | SM001T31.DAT |
| Advertisement | TWSWTCP F32.DAT 1 | SM001T32.DAT |
| Password | TWSWTCP F33.DAT 1 | SM001T33.DAT |
| Slaughter House | TWSWTCP F35.DAT 1 | SM001T35.DAT |
| Country | TWSWTCP F36.DAT 1 | SM001T36.DAT |
| Store Total | TWSWTCP F46.DAT 1 | SM001T46.DAT |
| System | TWSWTCP F48.DAT 1 | SM001T48.DAT |
| Cutting Hall | TWSWTCP F53.DAT 1 | SM001T53.DAT |
| Traceability | TWSWTCP F55.DAT 1 | SM001T55.DAT |
| Traceability Record | TWSWTCP F57.DAT 1 | SM001T57.DAT |
| Traceability Report | TWSWTCP F59.DAT 1 | SM001T59.DAT |
| Real Time Receipt Individual | TWSWTCP F64.DAT 1 | SM001T64.DAT |
| Real Time Receipt Total | TWSWTCP F65.DAT 1 | SM001T65.DAT |
| Print Format | TWSWTCP F74.DAT 1 | SM001T74.DAT |
| Text | TWSWTCP F78.DAT 1 | SM001T78.DAT |
| Scroll Message | TWSWTCP F79.DAT 1 | SM001T79.DAT |
| Scroll Sequence | TWSWTCP F70.DAT 1 | SM001T70.DAT |
| Preset Key | TWSWTCP F71.DAT 1 | SM001T71.DAT |
| Ingredient | TWSWTCP F60.DAT 1 | SM001T60.DAT |
| Special Message | TWSWTCP F61.DAT 1 | SM001T61.DAT |
| Shop Name | TWSWTCP F62.DAT 1 | SM001T62.DAT |
| Image (SM700) | TWSWTCP F11.DAT 1 | SM001T11.DAT |
| Department 2 (SM700) | TWSWTCP F13.DAT 1 | SM001T13.DAT |
| PLU 2 (SM700) | TWSWTCP F16.DAT 1 | SM001T16.DAT |

Note: It is NOT necessary to create the DOWNLOAD DATA FILE for each individual scale. The TWSWTCP.EXE driver will create the DOWNLOAD DATA FILE automatically during the DOWNLOAD operation. The driver will download the respective data files from each individual scale and store them in their respective DOWNLOAD DATA FILES.

##### ERROR CODES GENERATED BY TWSWTCP.EXE

The description of TWSWTCP Scale Driver Error Codes is stored in the RETVALS file and it is located in the same directory as the TWSWTCP scale driver. The list of Error Codes generated by TWSWTCP scale driver is as follows:

RETVALS FILE

| ERROR CODE | SYMBOL | DESCRIPTION |
| --- | --- | --- |
| -1 | # OPEN_FILE_ERR | Error opening input or output file |
| -2 | READ_FILE_ERR | Error reading input file |
| -3 | WRIT_FILE_ERR | Error writing to input or output file |
| -5 | NETWORK_OPEN_ERR | Error connecting to scale |
| -6 | NETWORK_READ_ERR | Error receiving data from scale |
| -7 | NETWORK_WRIT_ERR | Error sending data to scale |
| -8 | MACHINE_READ_ERR | Read error returned by scale |
| -9 | MACHINE_WRIT_ERR | Write error returned by scale |
| -10 | MACHINE_NOREC_ERR | No record error returned by scale |
| -11 | MACHINE_SPACE_ERR | No space error returned by scale |
| -12 | MACHINE_UNDEF_ERR | Undefined error returned by scale |

UPLOAD PLU ITEM FILE VIA TWSWTCP.EXE

The command to upload PLU Item file to scale with IP address 192.168.0.1 is as follows:

TWSWTCP F25.DAT 1

TWSWTCP driver will generate a Scale Number log file called ERROR after uploading. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

RETVALS log file will not be generated.

If ERROR file displays 1 means unable to upload PLU Item file to scale.

RETVALS file will be generated which shows the error code.

If RETVALS file displays –5 means unable to connect to scale.

It is necessary to create one PLU Item file (SM001F25. DAT).

The command to upload PLU Item file to two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F25.DAT 2 3

If ERROR file displays 2 means unable to upload PLU Item file to one scale only.

RETVALS file will be generated which shows the error code.

If RETVALS file displays –5 means unable to connect to one scale only.

If ERROR file displays 2;3 means unable to upload PLU Item file to both scales.

RETVALS file will be generated which shows the error codes.

If RETVALS file displays –5;–6 means unable to connect to scale with IP address 192.168.0.2 and unable to receive data from scale with IP address 192.168.0.3

It is necessary to create two PLU Item files (SM002F25.DAT & SM003F25.DAT).

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001F25.DAT ( STANDARD )

00000123002908000D2004000001801105200012300000000003070B526F6173746564205069670C0000000456003009008D2104000029501105200045600000000005000301070F4272756E2042616E616E61205069650C00

Sample PLU Item file of SM-80SX / SM-90 scale with IP address 192.168.0.1 for a single weighed PLU item and a single non-weighed PLU item. The sample PLU Item file consists of the following information for both PLU items:

Weighed PLU		Non-weighed PLU

-----------------		-----------------------

- PLU NUMBER			CHAR : 001 – 008	CHAR : 083 - 090

- PLU RECORD SIZE		CHAR : 009 – 012	CHAR : 091 - 094

- PLU STATUS			CHAR : 013 – 016	CHAR : 095 - 098

- PLU STATUS 2			CHAR : 017 – 022	CHAR : 099 - 104

- UNIT PRICE			CHAR : 023 – 030	CHAR : 105 - 112

- LABEL 1 FORMAT		CHAR : 031 – 032	CHAR : 113 - 114

- BARCODE FORMAT		CHAR : 033 – 034	CHAR : 115 - 116

- EAN DATA			CHAR : 035 – 048	CHAR : 117 - 130

- USED BY DATE			CHAR : 049 – 052	CHAR : 131 - 134

- QUANTITY						CHAR : 135 – 138

- QUANTITY SYMBOL					CHAR : 139 - 140

- COMMODITY NAME		CHAR : 053 – 080	CHAR : 141 - 176

- BCC				CHAR : 081 – 082	CHAR : 177 - 178

SM-80SX / SM-90 scale with IP address 192.168.0.1 will display the following information for the Weighed PLU item:

- PLU NUMBER			123

- PLU STATUS			WEIGH

- UNIT PRICE			$1.80

- LABEL 1 FORMAT		‘F1’ FREE FORMAT

- BARCODE FORMAT		FF  CCCCC  XXXXX  CD

- EAN DATA			FLAG : 20  &  ITEM CODE : 00123

- USED BY DATE			3 DAYS

- COMMODITY CHAR SIZE	M3

- COMMODITY NAME		Roasted Pig

SM-80SX / SM-90 scale with IP address 192.168.0.1 will also display the following information for the Non-weighed PLU item:

- PLU NUMBER			456

- PLU STATUS			NON-WEIGH

- UNIT PRICE			$29.50

- LABEL 1 FORMAT		‘F1’ FREE FORMAT

- BARCODE FORMAT		FF  CCCCC  XXXXX  CD

- EAN DATA			FLAG : 20  &  ITEM CODE : 00456

- USED BY DATE			5 DAYS

- QUANTITY			3

- QUANTITY SYMBOL TYPE	PCS

- COMMODITY CHAR SIZE	M3

- COMMODITY NAME		Brun Banana Pie

UPLOAD PLU DELETE FILE VIA TWSWTCP.EXE

The command to upload PLU Delete file to scale with IP address 192.168.0.1 is as follows :

TWSWTCP F95.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to upload PLU Delete file to scale.

It is necessary to create one PLU Delete file (SM001F95.DAT).

The command to upload PLU Delete file to two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F95.DAT 2 3

If ERROR file displays 2 means unable to upload PLU Delete file to one scale only.

If ERROR file displays 2;3 means unable to upload PLU Delete file to both scales.

It is necessary to create two PLU Delete files (SM002F95.DAT & SM003F95.DAT).

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001F95.DAT ( STANDARD )

000000030000002200000111

Sample PLU Delete file of SM-80SX / SM-90 scale with IP address 192.168.0.1 for three  PLU items. The sample PLU Delete file consists of the following information :

- PLU NUMBER 3			CHAR : 001 - 008

- PLU NUMBER 22		CHAR : 009 - 016

- PLU NUMBER 111		CHAR : 017 - 024

SM-80SX / SM-90 scale with IP address 192.168.0.1 will delete the following PLU items :

- PLU NUMBER			3

- PLU NUMBER			22

- PLU NUMBER			111

Note: The following command is used to delete ALL PLU items in the scale without creating a PLU Delete file.

TWSWTCP F88.DAT 1

DOWNLOAD PLU ITEM FILE VIA TWSWTCP.EXE

The command to download PLU Item file from scale with IP address 192.168.0.1 is as follows :

TWSWTCP F85.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to download PLU Item file from scale.

The command to download PLU Item file from two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F85.DAT 2 3

If ERROR file displays 2 means unable to download PLU Item file from one scale only.

If ERROR file displays 2;3 means unable to download PLU Item file from both scales.

Note: It is NOT necessary to create any PLU Item upload file SM001F85.DAT for any   SM-80SX / SM-90 scale. The sample PLU Item download file SM001T85.DAT consists of PLU Item information for all PLU items in the SM-80SX / SM-90 scale with IP address 192.168.0.1

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001T85.DAT ( STANDARD )

00000123002908000D2004000001801105200012300000000003070B526F6173746564205069670C0000000456003009008D2104000029501105200045600000000005000301070F4272756E2042616E616E61205069650C00009999980013740010000000000000FF099881009999990013750010000000000000FF099980E2

# The SM001T85.DAT download item file from scale has the same information as the SM001F25.DAT upload item file to scale except that the last byte of data E2 represents the

# End-of-Text. Furthermore, it consists of information for default items PLU 999998 and

# PLU 999999 too.

DOWNLOAD PLU TOTAL FILE VIA TWSWTCP.EXE

The command to download PLU Total file from scale with IP address 192.168.0.1 is as follows :

TWSWTCP F26.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to download PLU Total file from scale.

The command to download PLU Total file from two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F26.DAT 2 3

If ERROR file displays 2 means unable to download PLU Total file from one scale only.

If ERROR file displays 2;3 means unable to download PLU Total file from both scales.

Note: It is NOT necessary to create any PLU Total upload file SM001F26.DAT for any  SM-80SX / SM-90 scale. The sample PLU Total download file SM001T26.DAT consists of PLU Transaction information for those PLU items with sales transaction in the SM-80SX / SM-90 scale with IP address 192.168.0.1

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001T26.DAT ( STANDARD )

# 00000123006D20001230000000000997000001100000011000010000092600000110000001100000011000010000092600000110000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000240898

# 00000456006D20004560000000000997000008200000082000020000180600000820000008200000082000020000180600000820000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000240898E2

Sample PLU Total download file from SM-80SX / SM-90 scale with IP address 192.168.0.1 for two PLU items. The sample PLU Total download file consists of the following information :

PLU 123	PLU 456	End-of-Text

- PLU NUMBER			CHAR : 001 - 008	219 - 226 	437 - 438

- PLU TOTAL RECORD SIZE	CHAR : 009 - 012	227 - 230

- EAN CODE			CHAR : 013 - 026	231 - 244

- DUMMY				CHAR : 027 - 028	245 - 246

- MAIN GROUP CODE		CHAR : 029 - 032	247 – 250

DAILY TOTAL

- TOTAL ACTUAL PRICE	CHAR : 033 - 040	251 - 258

- TOTAL PLANNED PRICE	CHAR : 041 - 048	259 - 266

- TOTAL PCS			CHAR : 049 - 052	267 - 270

- TOTAL WEIGHT			CHAR : 053 - 060	271 - 278

- TOTAL PROFIT			CHAR : 061 - 068	279 – 286

TERM TOTAL

- TOTAL ACTUAL PRICE	CHAR : 069 - 076	287 - 294

- TOTAL PLANNED PRICE	CHAR : 077 - 084	295 - 302

- TOTAL PCS			CHAR : 085 - 088	303 - 306

- TOTAL WEIGHT			CHAR : 089 - 096	307 - 314

- TOTAL PROFIT			CHAR : 097 - 104	315 – 322

PREPACK DAILY TOTAL

- TOTAL ACTUAL PRICE	CHAR : 105 - 112	323 - 330

- TOTAL PLANNED PRICE	CHAR : 113 - 120	331 - 338

- TOTAL PCS			CHAR : 121 - 124	339 - 342

- TOTAL WEIGHT			CHAR : 125 - 132	343 - 350

- TOTAL PROFIT			CHAR : 133 - 140	351 – 358

PREPACK TERM TOTAL

- TOTAL ACTUAL PRICE	CHAR : 141 - 148	359 - 366

- TOTAL PLANNED PRICE	CHAR : 149 - 156	367 - 374

- TOTAL PCS			CHAR : 157 - 160	375 - 378

- TOTAL WEIGHT			CHAR : 161 - 168	379 - 386

- TOTAL PROFIT			CHAR : 169 - 176	387 – 394

DISCOUNT TOTAL

- TOTAL ACTUAL PRICE	CHAR : 177 - 184	395 - 402

- TOTAL PLANNED PRICE	CHAR : 185 - 192	403 - 410

- TOTAL PCS			CHAR : 193 - 196	411 - 414

- TOTAL WEIGHT			CHAR : 197 - 204	415 - 422

- TOTAL PROFIT			CHAR : 205 - 212	423 – 430

- LAST ACT DATE		CHAR : 213 - 218	431 - 436

# Note: The last byte of data E2 represents the End-of-Text.

UPLOAD PRESET KEY ASSIGNMENT FILE VIA TWSWTCP.EXE

The command to upload Preset Key Assignment file to scale with IP address 192.168.0.1 is as follows :

TWSWTCP F41.DAT 1

TWSWTCP driver will generate a log file called ERROR after communication. If an error occurs during uploading, an Error Code log file called RETVALS is also generated.

If ERROR file displays 0 means no communication error.

If ERROR file displays 1 means unable to upload PLU data file to scale.

It is necessary to create one Preset Key Assignment file (SM001F41.DAT).

The command to upload PLU data file to two scales with IP addresses 192.168.0.2 and 192.168.0.3 is as follows :

TWSWTCP F41.DAT 2 3

If ERROR file displays 2 means unable to upload PLU data file to one scale only.

If ERROR file displays 2;3 means unable to upload PLU data file to both scales.

Necessary to create two Preset Key Assignment files (SM002F41.DAT & SM003F41.DAT).

Note: Please refer to RETVALS FILE table on page 6 for the list of Error Codes generated by TWSWTCP scale driver.

SM001F41.DAT ( STANDARD )

0000000100130000012300504C5520313233200000000800130000045600504C552034353620

Sample Preset Key Assignment file of SM-80SX / SM-90 scale with IP address 192.168.0.1 for two PLU preset keys. The sample Preset Key Assignment file consists of the following information for both PLU preset keys:

Preset Key # 1		Preset Key # 8

-----------------		-----------------------

- PRESET NUMBER		CHAR : 001 – 008	CHAR : 039 - 046

- PRESET RECORD SIZE		CHAR : 009 – 012	CHAR : 047 - 050

- PRESET KEY SWITCH		CHAR : 013 – 020	CHAR : 051 - 058

- PRESET STATUS		CHAR : 021 – 022	CHAR : 059 - 060

- PRESET NAME			CHAR : 023 – 038	CHAR : 061 - 076

SM-80SX / SM-90 scale with IP address 192.168.0.1 will display the following information for both Preset Keys:

- PRESET KEY NUMBER		1

- PLU NUMBER			123

- PRESET KEY NAME		PLU 123

- PRESET KEY NUMBER		8

- PLU NUMBER			456

- PRESET KEY NAME		PLU 456